PRODUCT CARD STORK NEST SWING
Cat. no. 603

Swing dimensions:
Length: 3.8 m
Width: 2.3 m
Height: 2.2 m
Minimum space: 2.25 x 7.3 m
Age group: 0 to 14 years
Free fall height: 1.3 m

Guidelines for materials and technology of construction of the device.

The struts and the top beam, which form the swing structure, are made of 76.1 mm diameter stainless steel tube. The stainless steel is grade 0H18N9. The foundations are made as concrete footings founded at a depth of 0.6 m. The seat of the swing resembles a stork's nest. The frame is made of a metal circle with a diameter of 1.0 m, on which a rope is wound. The seat filling, in a shape resembling a spider web, is made of rope. The rope is reinforced with galvanised steel. A polypropylene braid provides a friendly surface for the child's hands. Fasteners, chains and slings are made of stainless steel. A maintenance-free roller bearing is used in the slings. End caps are made of plastic.

The fall area of the device should be made on a surface in accordance with EN 1176-1:2017.