PRODUCT CARD BELLATRIX
 Cat. no. 408

Device dimensions:

Length: 5.0m

Width: 0.95m

Height: 1.55m

Free fall height: 1,2m

Age group: 3 to 14 years old

Foundation depth: 1.2m

Minimum space: 6.0m x 5.2m

Amount of rope used: 8.2m

Guidelines for materials and technology of construction of the device.

BELLATRIX is a single device resembling a hammock. The supporting structure consists of steel poles protected against corrosion by hot-dip galvanizing. The poles are connected to the ground by reinforced concrete footings. The membrane is suspended from a braided polyamide rope reinforced with galvanized steel strings. The diameter of the rope is 18mm. The diaphragm is connected to the rope by a crossbar made of stainless steel, suspended from the rope by acid-resistant steel clamps. The membrane is reinforced with four layers of polyamide mesh. The device includes two ropes, connected to each other by aluminum crimps. The ropes are attached to the poles through bearing-mounted slings made of stainless steel. The sling has the ability to work in two directions. The connecting element between the ropes and the slings are stainless steel chains, forged to the ends of the ropes with an aluminum connector.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."