PRODUCT CARD JUMBO 
Cat. no. 411

Device dimensions:

Length: 7.1 m

Width: 5.1 m

Height: 3.0 m

Minimum space: 10.5 x 9.0 m

Age group: 5 to 14 years old

Foundation depth: 0.8m

Free fall height: 3.0 m

Guidelines for materials and technology of construction of the device.

JUMBO is a device consisting of a ball-shaped steel structure, a rope ball inside the steel structure, a rope jacket extending beyond the outline of the steel structure, and two rope floors. The steel structure is composed of four steel arches bent from 139.7-diameter pipes, connected to each other at 90 degrees. The steel structure is protected from corrosion by hot-dip galvanizing. The rope parts are made of polyamide, braided, glued ropes of 18mm diameter, connected by couplings made of aluminum, stainless steel and plastic. The tension of the rope part is provided by turnbuckles attached to the structure or anchored in the ground. The turnbuckles are protected against corrosion by hot-dip galvanizing. The foundation of the steel structure is made as reinforced concrete footings.

Optional painting.

The structure of the device at the request of the customer can be painted by powder or "wet" spray painting according to RAL.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."