PRODUCT CARD MIŁKUS
Cat. no. 606

Swing dimensions:
Length: 10.7 m
Width: 9.3 m
Height: 5.4 m
Minimum space: 12.0 m diameter circle
Age group: 0 to 14 years
Free fall height: 2.8 m

Guidelines for materials and technology of construction of the device.

The main structural element is a black steel tube with a diameter of 139.7 mm. The steel is protected against corrosion by hot-dip galvanising. The seat of the swing resembles a stork's nest and is attached to the structure by a two-articulated sling made of stainless steel. The frame is made of a metal circle with a diameter of 1.15 m, onto which a rope is wound. The seat padding, shaped like a spider web, is made of rope. The rope is reinforced with galvanised steel. A child-friendly surface for the hands is provided by a polypropylene braid. The foundations are made as concrete footings set at a depth of 0.6m. Fasteners, i.e. screws, bolts and screws made of stainless steel or protected against corrosion by galvanisation.

The fall area of the device should be made on a surface in accordance with EN 1176-1:2017.