PRODUCT CARD PALES 
Cat. no. 409

Device dimensions:

Length: 5.0m

Width: 5.0m

Height: 0.85m

Free fall height: 0,7m

Age group: 3 to 14 years old

Foundation depth: 0.8m

Minimum space: Ø 8.0m.

Amount of rope used: 36.0m

Guidelines for materials and technology of construction of the device.

PALES is a single baton-like device. The supporting structure consists of six steel poles protected from corrosion by hot-dip galvanizing. The poles are anchored in the ground with reinforced concrete footings. The rope structure consists of braided polyamide ropes reinforced with galvanized steel strings, and fasteners made of polyamide, aluminum and stainless steel. The diameter of the rope is 18mm. A rubber membrane in the shape of a circle with a diameter of 2.0m is connected to the rope structure with polyamide cautions. The membrane is made of rubber mat reinforced with four layers of polyamide mesh. The rope structure is connected to the poles with stainless steel cautions, mounted on stainless steel cybants, allowing the device to be stretched.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."