PRODUCT CARD PROCJON 
Cat. no. 403

Device dimensions:

Length: 1.8 m

Width: 1.8 m

Height: 3.0 m

Minimum space: a circle with a diameter of 5.5 m

Age group: from 3 to 14 years old

Foundation depth: 0.8 m

Free fall height: 2.0 m

Guidelines for materials and technology of construction of the device.

The main structural element is a steel pole with a diameter of 139.7 mm protected from corrosion by hot-dip galvanizing. The cone shape is obtained by tensioning the ropes on a 0H18N9 grade stainless steel ring. The ring has a diameter of 1800 mm and is made of 42.4 mm diameter pipe. The rope used is 16 mm in diameter, reinforced with steel and made of polypropylene. The elements connecting the ropes to each other are made of plastic and aluminum. The elements connecting the ropes to the pole are made of stainless steel.

Optional painting.

The structure of the device at the request of the customer can be painted by powder or "wet" spray painting according to RAL.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."