PRODUCT CARD ARCHER 
Cat. no. 420

Device dimensions:

Length: 6.2 m

Width: 4.3 m

Height: 2.5 m

Minimum space: 9.5 x 7.8 m

Age group: 3 to 14 years old

Foundation depth: 0.8m

Free fall height: 2.5 m

Guidelines for materials and technology of construction of the device.

The device consists of two steel arches with a diameter of 139.7 mm anchored in the ground with reinforced concrete feet, on which a rope structure in the shape of an hourglass is tensioned. The steel elements are protected from corrosion by hot-dip galvanizing. The three outermost ropes (looking from the bottom of the structure) are connected to the structure by clamps made of cast steel (number of clamps: 12 pieces). This solution is intended to introduce hinges in places where forces act in multiple directions. The bracket will be protected against corrosion by painting with chlorine rubber paint. The turnbuckles for tension correction are protected against corrosion by hot-dip galvanizing. The rope used is 18 mm in diameter and is braided, glued, reinforced with steel and made of polyamide. The elements connecting the ropes to each other are made of polyamide, aluminum or stainless steel. An additional attraction is the five climbing ropes. Each rope is equipped with three plates.

Optional painting.

The structure of the device at the request of the customer can be painted by powder or "wet" spray painting according to RAL.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."