PRODUCT CARD JUMPER
 Cat. no. 419

Device dimensions:

Length: 5.4m

Width: 4.7m

Height: 1.2m

Minimum space: ø 7.2m

Age group: 3 to 14 years old

Foundation depth: 0.8m

Free fall height: 1.2m

Guidelines for materials and technology of construction of the device.

The device consists of six poles made of black steel with a diameter of 101.6 mm and a rope net. The poles are founded in the ground with reinforced concrete feet. The connection of the poles to the feet is made as unidirectionally articulated. Lashings of poles anchored in the ground with reinforced concrete feet. Elements made of black steel are protected against corrosion by hot-dip galvanizing. The rope used in the production has a diameter of 18 mm and is braided, glued, reinforced with steel and made of polyamide. The elements connecting the ropes to each other are made of polyamide, aluminum or stainless steel. The rope net is equipped with rubber of min. 9mm, reinforced with three layers of polyamide mesh. The tension of the net is provided by turnbuckles anchored in the ground with reinforced concrete feet.

Optional painting.

The structure of the device at the request of the customer can be painted by powder or "wet" spray painting according to RAL.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."