PRODUCT CARD TOPIK 
Cat. no. 424

Device dimensions:

Length: 7.3m

Width: 6.9m

Height: 4.1m

Minimum space: 11.1m x 9.8m

Age group: 5 to 14 years old

Foundation depth: 1.0 m

Free fall height: 3.0 m

Guidelines for materials and technology of construction of the device.

TOPIK is a device with a steel structure in the shape of an inclined dome, inside which is suspended a spatial net of irregular shape. The eyes of the net are shaped like cuboids. The individual walls of the net are filled with alternating denser nets, creating a rope maze with an infinite number of combinations of passages.  The nets are tensioned by hot-dip galvanized turnbuckles anchored in the ground.

The steel structure made of black steel is protected from corrosion by hot-dip galvanizing and powder coating. Rope elements are made of polyamide, braided, glued ropes with a diameter of 18mm, connected with connectors made of aluminum, stainless steel and plastic. Connection of the rope net to the clamps is provided by stainless steel eyebolts. The tension of the rope part is provided by hot-dip galvanized turnbuckles. The foundation of the steel structure is made as reinforced concrete footings.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."