[image: image1.jpg]T

T

i






INDIVIDUAL PROJECT
TREES cat. nr 1242
[image: image2.jpg]



Dimensions of the device: Length: 11.8 m

Width: 3.8 m

Height: 6.8 m

Free fall height: 2,0 m

Age group: 5 to 14 years old

Foundation depth: 1,0 m
Minimum space: 15,2 x 6,8 m[image: image3.jpg]6,8m

3,8m

il
OO

IDOIDO!
MO

IDCNIDCICD
(I
DO
DD

e
IO

(DCDLIL)
DOODOIDX
IDODCD!
MDD

weL 1

we'gl




[image: image4.png]*mogicnel:s 6 +48 71 340 09 19 e +48 71 341 62 63 a biuro@magicnets.pl é www.magicnets.pl o UL. Transportowa 4, 55-020 Zérawina

MagicNets Sp. z 0.0. | NIP: 8992655629 | Kapitat zaktadowy: 50 000,00 zt | Sad Rejonowy dla Wroctawia Fabrycznej we Wroctawiu VI Wydziat Gospodarczy Krajowego Rejestru Sadowego | KRS: 0000319293




Device Description:

The main structural element is two steel towers with a total height of 6.8m. The core of the structure consists of 120 x 120mm square steel pipes. The structure is hot-dip galvanized and powder-coated. The towers, imitating coniferous trees, are decorated with HPL panels of external application and 10mm thick.   Four windows made of transparent polycarbonate are installed in each tower.  Additional decoration is provided by animals located in the windows, made by printing on polycarbonate. Inside each tower are made of ropes: 2 full floors and 3 mezzanines allowing communication inside the toy, between levels. Movement between the towers is made possible by a roped, openwork tunnel with a cross section of 1.9x1.0m and a length of 5.0m. The bottom of the tunnel is suspended at a height of 3.5m. Entrance from the outside to the tower is provided by two entrances: a steel-rope arch entrance and a rope entrance in the form of a sloping trapezoidal truss. The net is made of polyamide rope, braided, glued reinforced with galvanized steel strings. The diameter of the rope is 18 mm. The elements connecting the ropes to each other are made of plastic and aluminum. 
The fall area of the device should be made on the surface in accordance with PN EN 1176- 1:2017-12. The possibility of access of heavy construction equipment is required for the installation.

For the sake of children's safety and the quality of the equipment, it is required that the equipment is certified in accordance with the standards PN EN 1176-1:2017-12, PN EN 1176-11:2014-11 issued in a system accredited by the National Accreditation Center or the national accreditation body of the other Member States, in accordance with the Regulation of the European Parliament and the Council of the European Union (EC) No 765/2008."
