[image: image1.jpg]




[image: image2.jpg]


PRODUCT CARD STICKER
[image: image3.jpg]13m

10,1m




Cat. no. 1632
[image: image4.png]*mogicnel:s 6 +48 71 340 09 19 e +48 71 341 62 63 a biuro@magicnets.pl é www.magicnets.pl o UL. Transportowa 4, 55-020 Zérawina

MagicNets Sp. z 0.0. | NIP: 8992655629 | Kapitat zaktadowy: 50 000,00 zt | Sad Rejonowy dla Wroctawia Fabrycznej we Wroctawiu VI Wydziat Gospodarczy Krajowego Rejestru Sadowego | KRS: 0000319293




Device dimensions: 

Length: 12.5 m

Width: 10.1 m

Height: 9.0 m

Minimum space: 16.0 x 13.0 m 

Age group: 5 to 14 years old

Free fall height: 3.0 m

Foundation depth: 1.0 m
Guidelines for materials and technology of construction of the device.

STICKER is a device with the character of an observation tower, looking like a stork's nest on sailing ships. The device forms a centrally located pole with a diameter of 219.1mm. At a height of 3m is a platform with railings, with a steel structure and a diameter of 1.9 m. The platform and railings are made of HPL boards. On the platform is the starting part of a straight tube slide made of stainless steel. Around the central pole are located 4 steel poles. Between the poles and the platform is stretched a three-armed rope net. At the bottom, two hammocks made of ropes are attached to the net.
Steel components protected from corrosion by hot-dip galvanizing and powder coating. The rope elements are made of polyamide, braided, bonded ropes with a diameter of 18mm, connected by connectors made of aluminum, stainless steel and plastic. The connection of the rope net with the steel structure is ensured by elements made of stainless steel, galvanized steel and plastics. The foundation of the steel structure is made of reinforced concrete footings.
The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."
