Product card SWING PORTIA

Cat. no. 615

Swing dimensions:

Length: 3.5 m

Width: 2.2 m

Height: 2.5 m

Minimum space: 2.2 x 8.0 m

Age group: 5 to 14 years

Free fall height: 1.8 m

Guidelines for materials and technology of construction of the device.

The struts and top beam, which form the swing structure, are made of 76.1 mm diameter stainless steel tube. The stainless steel is grade 0H18N9. The foundations are made as concrete footings placed at a depth of 0.6 m. Seat designed for a parent with a young child, sitting facing each other. Seat frame made of stainless steel. A „board” seat (for the parent) and a „diapers” seat (for the child) are hanging to the frame. Both seats are made of a steel insert covered with EPDM rubber. Connectors, chains and slings are made of stainless steel. A maintenance-free rolling bearing is used in the slings. The plugs are made of plastic.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."