[image: image1.png]




PRODUCT CARD VIL 3
[image: image2.png]


Cat. no. 1416
Device dimensions: 

Length: 28.3 m

Width: 13.5 m

Height: 2.6 m

Minimum space: 31.3 x 18.1 m

Age group: 5 to 14 years old

Free fall height: 2.5 m

Foundation depth: 1.0 m
[image: image3.jpg]C3,3M

SSEN %
90%%%%%
(XK

SEERRHNS
SEISIRRLIKS

RS

, &
, 20
02070 3958





[image: image4.png]*mogicnel:s 6 +48 71 340 09 19 e +48 71 341 62 63 a biuro@magicnets.pl é www.magicnets.pl o UL. Transportowa 4, 55-020 Zérawina

MagicNets Sp. z 0.0. | NIP: 8992655629 | Kapitat zaktadowy: 50 000,00 zt | Sad Rejonowy dla Wroctawia Fabrycznej we Wroctawiu VI Wydziat Gospodarczy Krajowego Rejestru Sadowego | KRS: 0000319293




Guidelines for materials and technology of construction of the device.

The main structural element is a 168.3mm diameter steel pipe and 139.7mm diameter vertical poles. Structure protected against corrosion by hot-dip galvanizing. The device is anchored in the ground with reinforced concrete footings. The nets are made of polyamide rope, braided, glued reinforced with galvanized steel strings. The diameter of the rope is 18 mm. The connectors used are made of polyamide. Press-forged parts made of aluminum. Connection of ropes to the steel structure by : hot-dip galvanized shackle, stainless steel thimble. The average mesh eye size is 350 x 350 mm. Additional attractions include:
- 3 climbing ropes, equipped with plastic handles,
- 3 “Stork Nest” type seats with a diameter of 950 mm,

- 2 plates installed to vertical poles,

- large rubber mat, attached to the sloping part of the structure, allowing climbing and sliding. 8mm thick rubber mat, reinforced with four layers of polyamide mesh,
- 8 rubber mats of different shapes and sizes.  8mm thick rubber mat, reinforced with four layers of polyamide mesh,
- one end equipped with a dense net with a medium mesh size of 120 x 120mm.
Optional painting.

The structure of the device at the request of the customer can be painted by powder or “wet” spray painting according to RAL.
The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."
