[image: image1.png]




PRODUCT CARD DIA
Cat. no. 220
[image: image2.jpg]O,1m

S, 9m

0,9m





Device dimensions:

Length: 5.9 m

Width: 5.1 m

Height: 4.5 m

Minimum space: 8.9 m diameter circle

Age group: 3 to 12 years

Free fall height: 1.0 m

Foundation depth: 0.7 m
[image: image3.png]*mogicnel:s 6 +48 71 340 09 19 e +48 71 341 62 63 a biuro@magicnets.pl é www.magicnets.pl o UL. Transportowa 4, 55-020 Zérawina

MagicNets Sp. z 0.0. | NIP: 8992655629 | Kapitat zaktadowy: 50 000,00 zt | Sad Rejonowy dla Wroctawia Fabrycznej we Wroctawiu VI Wydziat Gospodarczy Krajowego Rejestru Sadowego | KRS: 0000319293




Guidelines for materials and technology of construction of the device.

The main structural element is a 4.5-metre steel pole with a diameter of 139.7 mm, which is protected against corrosion by hot-dip galvanising and powder coating. The rope structure consists of six main ropes anchored to the ground with reinforced concrete footings. Tension adjustment is made possible by hot-dip galvanised turnbuckles. Six rope walls are stretched between adjacent supporting ropes. Additional attractions include horizontal rope levels at heights of 1.0 and 1.8 m. The net is made of polypropylene rope, twisted, reinforced with galvanised steel strings. The diameter of the rope is 16 mm. The elements connecting the ropes to each other are made of plastic and aluminium. The elements connecting the ropes to the pole are made of stainless steel.
The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017-12. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017-12, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."
